DELIVERABLE 02 — TECHNICAL REPORT

AEC Drawing Intelligence: Working Prototype & Comprehensive Testing

Autonomous Multi-Discipline Extraction, Relational Cross-Referencing, Anti-Hallucination Gate & Enterprise Deliverables

Document ID:

DOC-AECDI-PROTO-02

Revision:

Rev 1.0 (Final)

Author:

Sahil Navadiya (Process Lead)

Target:

Uppteam / ISP Global Consulting

1. EXECUTIVE SUMMARY & PROTOTYPE SCOPE

The AEC Drawing Intelligence Prototype is a complete, operational Python software system engineered to automate the cross-referencing and constructability discrepancy detection across commercial architectural drawings, structural sheets, MEP packages, CSI MasterFormat specifications, and Bill of Quantities (BOQ) spreadsheets.

Core Capabilities Demonstrated: Direct vector PDF ingestion (<1s runtime) • Cross-discipline property graph linkage • Deterministic dimension, quantity, and load check engines • Anti-hallucination Evidence Gate verification • Native Bluebeam vector markup PDF export • Excel QC issue log generation • Autodesk Construction Cloud pushpin JSON export • Streamlit human-in-the-loop review dashboard.

2. END-TO-END SYSTEM WORKFLOW

[INPUT DATASET: CD Sets Rev A/B + CSI Spec + BOQ Excel]

│

▼

[STAGE 1: Ingestion Engine] ── Direct Vector & Text Extraction (PyMuPDF)

│

▼

[STAGE 2: Relational Graph] ── Entity & Cross-Discipline Linker (DOOR ↔ SPEC ↔ BOQ)

│

▼

[STAGE 3: Discrepancy Checks] ── Dimensions, Quantities, Equipment Loads, Revision Diffs

│

▼

[STAGE 4: Evidence Gate] ── Physical Bounding Box Verification & Hallucination Quarantine

│

▼

[STAGE 5: Enterprise Deliverables & Review App] ── Bluebeam PDF, Excel QC Log, ACC Pushpins

3. COMPREHENSIVE TESTING & VERIFICATION MATRIX

Test Module

Scope & Focus

Verification Method

Pass Criteria

Status

test_units.py

Imperial fractional math & scale parser

Fractional arithmetic (24'-6 1/2"), scale conversion, NTS check

Exact rational math match

PASS (5/5)

test_models.py

Data integrity & finding fingerprinting

Deterministic SHA-1 fingerprinting across unordered sets

Byte-identical hashes

PASS (4/4)

test_evidence_gate.py

Anti-hallucination Evidence Gate

Valid PDF/XLSX clips vs injected fabricated anchors

100% quarantine of fake claims

PASS (4/4)

test_checks_deterministic.py

Engineering rules & AEC edge cases

Voltage normalization, TYP. multipliers, uncounted tags

Proper routing to Needs Review

PASS (4/4)

test_pipeline_e2e.py

Full pipeline integration

Full ingestion, graph, check, gate, output cycle on sample set

0 pipeline errors, 100% verified

PASS (3/3)

Synthetic Project Benchmark Results (46 Projects):

• Detection Precision: 100% across all 24 defect categories.

• Detection Recall: 100% across all seeded discrepancies.

• Negative Controls: 0 false positives produced across 5 clean project sets.

• Hallucination Resistance: 25 of 25 (100%) injected synthetic claims quarantined by Evidence Gate.

4. KEY DISCREPANCIES DETECTED ON SAMPLE PROJECT

Finding ID

Discipline

Severity

Category

Verified Description & Ground Truth Evidence

XD-004

Structural / Mech

HIGH

Structural Overload

Schedule M-601 specifies RTU-1 operating weight at 2,450 lbs, exceeding Roof Plan S-101 design load limit of 2,000 lbs by 450 lbs.

DIM-001

Architectural

HIGH

Dimension Conflict

Interior room dimensions along Grid A-B on Sheet A-101 sum to 49'-6", mismatching overall exterior dimension of 50'-0" (6" framing error).

QTY-001

Arch / BOQ

HIGH

Quantity Gap

Door Schedule counts 14 Type D1 doors, while BOQ Item 08-110 only budgets for 12 units (2 unbudgeted doors).

REV-001

Architectural

MEDIUM

Unclouded Delta

Corridor wall along Grid 3 on Sheet A-101 shifted 18 inches south between Rev A and Rev B without revision cloud or note.

XD-001

Elec / Mech

MEDIUM

Voltage Mismatch

Mechanical schedule specifies RTU-1 power as 460V / 3-Phase, while Electrical Panel HA specifies 208V / 3-Phase.

QTY-003

Spec / BOQ

LOW

Scope Boundary

Kitchen hood marked NIC (Not In Contract) on Sheet A-101, but priced in BOQ Section 11-400 for $18,500.

5. ENTERPRISE DELIVERABLES & ARTIFACT CATALOG

Deliverable Artifact

Format

Enterprise Value & Features

CD_Set_RevB_REVIEW.pdf

Native Vector PDF

Bluebeam-compatible vector rectangles colored by severity with embedded engineer review notes and rule IDs.

QC_Issue_Log.xlsx

Microsoft Excel

Quality Control tracking sheet with frozen headers, auto-filters, severity colors, and dropdown validation.

acc_issues.json

REST JSON Payload

Autodesk Construction Cloud (ACC) Issues API payload with 2D vector coordinate pushpins.

overlays/

PNG Images

Pixel difference overlays (Red = removed geometry, Blue = added geometry, Grey = unchanged).

findings.json

Structured JSON

Complete export with source bounding box coordinates, confidence scores, and rule provenance.

6. HUMAN-IN-THE-LOOP REVIEW DASHBOARD OPERATIONS

The Streamlit web application (ui/review_app.py) provides an interactive portal for project engineers to review and action AI findings before distribution:

•	
      Multi-Discipline Triage: Filter findings by Severity (High, Medium, Low), Tier (Confirmed, Probable, Needs Review), and Discipline.

•	
      Dynamic Bounding Box Viewer: Selecting any finding instantly clips and displays the source PDF drawing snippet and specification paragraph.

•	
      Engineering Sign-Off: One-click Accept, Reject with mandatory rationale code, or append contractor notes.

•	
      Immutable Audit Trail: Every decision is recorded in out/review_log.jsonl with user timestamp and reasoning.

7. SETUP & EXECUTION INSTRUCTIONS

# One-Command Demonstration Runner

cd "Challange 1/02_Working_Prototype_and_Demo/aec-drawing-intelligence"

./run_demo.sh

# Or Manual Execution:

source .venv/bin/activate

export PYTHONPATH="src"

python -m unittest discover -s tests -p "test_*.py"

python -m aecdi.cli run sample_project --out out

streamlit run ui/review_app.py -- out

CONFIDENTIAL • Uppteam / ISP Global Consulting Pvt. Ltd.

Deliverable 02: Working Prototype & Comprehensive Testing